Some notes on the programs contained on this disk...

NEOchrome:

This is a version of our favorite ST graphics program with a new

feature added.  In addition to the "Screen" (320x200), a new graphics

region known as the "Canvas" (640x400) is now available for use.  By using

the grabber hand, one can position any portion of the canvas to appear on

the screen.  Since the canvas is twice as wide & twice as high as the

screen, four times the previous picture area may be used for graphics

generation.  This feature was requested by some of the coin-op game artists

working at Atari Games at the time.  It should be noted that when saving a

picture, you will be asked to specify "Screen" or "Canvas".  Now you'll

know what is meant.

Another feature present in version 1.0 (as well as this version) which

was hidden & undocumented, is "Animate".  This feature was hidden because I

wasn't able to finish it to my satisfaction and hence it is somewhat of a

kludge.  However, you may find it useful so I will reveal how to invoke

"animate":  Get into grabber hand mode to make the word GRABBER appear on

the right side.  Position the mouse pointer so that it points to the

"armpit" of the second "R" in "GRABBER".  It should be exactly within the

inside portion of the top/left vertex, then click with the right mouse

button.  The "Animate" icon will appear in the tool grid on the left side.

Click on this icon, and NEOchrome will enter animate mode.

Once in this mode, begin by creating a box in the drawing area

somewhat like you would if using "COPYBOX".  This "box" defines the cell

size desired.  Note that the width is forced to 16-pixel multiples and the

left origin is similiarly constrained.  Now click "ADD" once for each cell

desired in the animate sequence.  The current cell number is indicated in

the center rectangle.  Click on the right arrow-box to step once forward,

and click on the left arrow-box to step once backward.  Now, to load

graphics from the NEOchrome screen "underneath" to the current cell, click

with the right button when the mouse is anywhere on the drawing area and

"drag" the graphics until positioned as desired.  Then step to the next

cell to record these graphics to this cell number.  Load up the cells with

graphics in this manner until all the cells have graphics.  Click the right

arrow-box with the right mouse button to start animate.  Speed up and slow

down the rate by right or left clicks on the arrow-box in a way similiar to

how color scroll animation is controlled.  To stop, click on the opposite

arrow-box.

Be careful when animating at full speed (60 hz) not to move the mouse

off the arrow-box (since the mouse disappears).  Slow down before moving

the mouse to the opposite arrow-box to stop animation.  Once a cell is

loaded with graphics, fine positioning within the cell is accomplished with

the left mouse button when the cursor is anywhere on the drawing area.  The

right mouse button reloads from the NEOchrome picture underneath as

described above.  Click on the small "Copybox" icon in the lower right part

of the animate controller area to copy all graphics contained in the

current cell to the Copybox cut buffer (accessable from Copybox or

Jacknife).

The animate feature is not polished and has a few bugs to watch out

for.  Do not use UNDO or FULL SCREEN while in animate and be sure to "step

1

off" a cell after loading it with graphics to record the graphics to that

cell.  "Load", "Save", and "New" are straightforward file functions.

Animate files are given the .ani extension.  It is useful to create

NEOchrome screens with cell grids containing animation graphics.  When it

becomes necessary to change graphics for a cell, the changes can be made

within the cell grid and "re-loaded" into the animate cell.       Please do

not distribute this version of NEOchrome as it is copyrighted and

serialized.

TEMPLATE.NEO:

This is a NEOchrome picture file with the template used for 4k

graphics editting.  The program NEO2S.TTP described below expects as input

a NEOchrome format picture file with this template filled with graphics.

The graphics to be used here are for 160x2 mode on the 7800.  Other less

widely used modes will be supported at a later date.  Only the first 4

colors of the NEOchrome color palette are used and each 7800 pixel is

represented by 2 NEOchrome pixels aligned horizontally with the leftmost

NEO pixel on an even x-postion.  One can think of the 4k graphics block

edittable with this system as a horizontal strip of graphics 1024 pixels

wide by 16 pixels high.  Since each 160x2 mode graphics byte contains 4

pixels, 1024 pixels are found in each page of 6502 memory which make up one

horizontal line of graphics on the NEOchrome template.  16 pages (4k) are

required for the 16 horizontal lines represented on this template.  Byte

boundaries are indicated by the vertical blue lines.  Only 32 bytes of 7800

style graphics can conveniently fit within the horizontal confines of the

NEOchrome screen, so 8 such strips were used to represent what should be

thought of as a single very wide strip 1024 pixels wide by 16 lines high.

Each graphics object needs a label for referencing within a

developers' program.  Since objects can vary in width from 1 to 32 bytes

(always assuming 16 lines high), a method is needed to indicate where one

object ends and another begins.  Use the right-most color (color 15,

usually white) on the NEOchrome palette to draw with the pencil tool a

single pixel on top of the blue byte boundary immediately above the

enclosed graphics strip.  An example of this dot placement is found on the

template.  The dot should be placed on the byte boundary between any two

graphics objects to delimit them.  This will tell NEO2S.TTP where the label

breaks should be.  The left-most byte is assumed to be the start of a

graphics object and needn't have such a pixel indicator.

Once you have completed your graphics creation on this template, save

it (as a screen, not a canvas) being careful that the screen is zero-

origined within the canvas.  This means that the top/left corner of the

screen is exactly the top/left corner of the canvas.  Now you are ready to

create a graphics source file using the next utility described below...

2

NEO2S.TTP:

This program is pretty simple; just give it the name of an input .NEO

file on its command line, enter the name of an output .S file when prompted

for it (or just <cr> if the input file name with .S appended should be

used), and voila!--you have a graphics source file.  Only graphics found

within the bounds of the template are used with the 4-color/2 pixel

restrictions applying as described above.  The source file is MADMAC

compatible and generates 4k of graphics in 7800 160x2 mode with labels

provided to delimit each graphics object.

Other programs and files may be found on this disk which should have

accompanying README type documents describing their use.

If there are any questions, you may call Dave Staugas at [408] 745-2267

3